Use InnoScan AL1100 Scanner to Scan Microarray Slide

General Procedure:

Scan slides:

- Turn on scanner, open Mapix software, select the “Connect to Scanner” button and wait for the laser to warm up for at least 5 min.

- Insert dry slide into the slide loader, click the “Insert” button and the scanner detects how many slides are loaded and corresponding position. Select the slides you want to scan.

- Set scan configuration. Below parameters are standard settings used by our core.

- Click the “Scan” button and the software promotes to ask where to save the images.  Choose the appropriate folder and enter file name.  Scanned image is saved automatically as a TIF file.  Below options are our standard settings:

- Use TIFF LZW compression (lossless);

- Prefix file name: Date & Time.

- After scanning is done, click the “Disconnect to Scanner” button and turn off the scanner power.

Fluorescent Signals Acquisition and Data Analysis

- When quantifying the fluorescent signals, open the image TIF file in Mapix software.

- Load the Grid file (or GAL file), align the spots using automatic find function, and then manually align blocks and spots.

- Set Algorithms parameters as shown below.

- Save result with the following options:

1) All blocks in a file;

2) Save a JPEG file;

3) Integer value.

- A raw data file containing quantified fluorescent signal is a file with .gpr extension and it is a txt file. Use Microsoft excel to open it and process the data. Alternatively, import them to other data analysis software such as R to do the downstream data summary.